WLAN-2450

2.4/5 GHz Dual-Band Sub-Sampling WLAN Receiver

| DESCRIPTION The WLAN-2450 is a fully integrated dual-band sub-sampling receiver front-end supporting operation in the 2.4 GHz and 5 GHz WLAN bands. The receiver employs a direct RF-to-IF conversion architecture with optimized sub-sampling frequency selection to achieve high receiver performance while reducing analog front-end complexity and power consumption. The device integrates an active balun low-noise amplifier (LNA), sub-sampling down-conversion mixer, intermediate-frequency (IF) amplifier, and switched-capacitor decimation filter within a single receiver chain. The optimized architecture supports continuously programmable gain and dual-band operation from 2.4 GHz to 5.7 GHz, enabling an error vector magnitude (EVM) of −40 dB over a 19-dB input power range. Implemented in a 65-nm CMOS process and occupying only 0.72 mm², the WLAN-2450 consumes 55.6 mW from a 1.2-V supply while supporting signal bandwidths of up to 77 MHz. The receiver is ideally suited for IEEE 802.11 WLAN systems, wireless access points, embedded Wi-Fi modules, software-defined radio (SDR) platforms, and other dual-band wireless communication applications. | KEY FEATURES - Dual-band operation: 2.4 GHz and 5 GHz WLAN - Direct RF-to-IF sub-sampling receiver architecture - Active balun low-noise amplifier (LNA) - Sub-sampling down-conversion mixer - Switched-capacitor decimation filter with integrated band selection - Continuously tunable gain: 26–41 dB (2.4 GHz), 26–38.5 dB (5 GHz) - Continuously tunable 5-GHz band (4.5–5.7 GHz) - Signal bandwidth: 77 MHz - Error Vector Magnitude (EVM): −40 dB - Noise Figure: 11.5 dB - IIP₃: −8 dBm - Power consumption: 55.6 mW - Active area: 0.72 mm² in 65-nm CMOS - Supply voltage: 1.2 V (core) |
| --- | --- |

APPLICATIONS

| Application | Relevant Bands | Notes |
| --- | --- | --- |
| Wi-Fi Access Points | 2.4 GHz / 5 GHz | Dual-band WLAN operation |
| Wireless Client Devices | 2.4 GHz / 5 GHz. | High-performance receiver front-end |
| Embedded Wi-Fi Modules | 2.4 GHz / 5 GHz | Compact, low-power integration |
| Software-Defined Radio (SDR) | 2.4–5.7 GHz | Reconfigurable sub-sampling receiver |
| Industrial Wireless Networks | 2.4 GHz / 5 GHz | Wideband wireless connectivity |

ELECTRICAL SPECIFICATIONS

TA = 27°C, VDD = 1.2 V, unless otherwise noted.

| Parameter | Symbol | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| RF Frequency Range | fRF | 2.4 | — | 5.7 | GHz | Dual-band operation |
| IF Frequency | fIF | 445 | — | 538 | MHz | Direct RF-to-IF conversion |
| Signal Bandwidth | BW | — | 77 | — | MHz | WLAN signal bandwidth |
| Receiver Gain (2.4 GHz) | G | 26 | — | 41 | dB | Continuously programmable |
| Receiver Gain (5 GHz) | G | 26 | — | 38.5 | dB | Continuously programmable |
| Error Vector Magnitude | EVM | — | −40 | — | dB | Over 19-dB input power range |
| Noise Figure | NF | — | 11.5 | — | dB | Highest gain setting |
| Input IIP₃ | IIP₃ | — | −8 | — | dBm | Lowest gain setting |
| Power Consumption | P | — | 55.6 | — | dB | Total receiver power |
| Active Area |  | — | 0.72 | — | mm² | 65-nm CMOS |
| Supply Voltage (Core) | VDD | — | 1.2 | — | V | Core supply |

POWER CONSUMPTION

Note: Power consumption is identical for both the 2.4 GHz and 5 GHz receive modes.

| Block | Power | Supply | Notes |
| --- | --- | --- | --- |
| Active Balun LNA | 19.88 mW | 1.2 V | Dual-band low-noise amplification |
| IF Amplifier | 5.39 mW | 1.2 V | Programmable IF gain stage |
| Baseband Amplifier | 2.52 mW | 1.2 V | Baseband signal amplification |

ARCHITECTURE OVERVIEW

| Receiver Signal Path - Active balun LNA provides low-noise differential RF amplification. - Sub-sampling mixer performs direct RF-to-IF down-conversion. - IF amplifier provides continuously programmable receiver gain. - Switched-capacitor decimation filter performs baseband conversion and channel selection. - Supports dual-band operation at 2.4 GHz and 5 GHz WLAN bands. | Gain Control & Band Selection - Continuous receiver gain tuning for improved EVM performance. - Supports 26–41 dB gain (2.4 GHz) and 26–38.5 dB gain (5 GHz). - Continuously tunable 5-GHz operating band from 4.5–5.7 GHz - Optimized sub-sampling frequency minimizes receiver impairments. - Achieves −40 dB EVM over a 19-dB input power range. |
| --- | --- |

FREQUENCY PLAN

| Parameter | Value |
| --- | --- |
| RF Frequency (fRF) | 2.4 GHz / 4.5–5.7 GHz |
| Sampling Frequency (fS) | 1.78–2.15 GHz |
| Intermediate Frequency (IF) | 445–538 MHz |
| Signal Bandwidth | 77 MHz |

ABSOLUTE MAXIMUM RATINGS

Stress above listed limits may permanently damage the device. Functional operation at these limits is not implied.

| Parameter | Limit | Unit |
| --- | --- | --- |
| Core Supply Voltage (VDD) | 1.2 + 0.3 | V |
| I/O / Buffer Supply Voltage | 2.5 + 0.5 | V |
| Operating Temperature | −40 to +85 | °C |
| ESD Rating (HBM) | 2000 | V |

DEVICE INFORMATION

| Parameter | Value |
| --- | --- |
| Part Number | WLAN-2450 |
| Technology | 65 nm CMOS (TSMC 1P9M) |
| Die Size (Active) | 0.72 mm² |
| Die Supply (Core) | 1.2 V |
| Die Supply (I/O) | 2.5 V |
| Package | QFN40 |
| Operating Temperature Range | −40°C to +85°C |

DESIGN SUPPORT & RELATED RESOURCES

For more information, visit https://www.linkedin.com/company/chipinsemi/ All product names, logos, and brands are property of their respective owners